Installation Guide of PMC RAID Card Driver in VMware System

目录

1.	Precautions for operation	3

2.	Preparation before operation	3

3.	Driver Installation	3

4.	Driver Uninstallation	4

Precautions for operation

- This driver version is only applicable to VMware ESXI 6.7.

- This driver is suitable for PMC RAID cards.

- Driver upgrade requires server restart to take effect.

Preparation before operation

- Query the current operating system kernel.

#uname -a

- Query the current driver version. The driver version is 1.0.1.553.

#esxcli software vib list|grep smartpqi

Driver Installation

- After decompressing the driver package, upload the driver file MSCC_bootbank_smartpqi_67.4252.0.103-1OEM.670.0.0.8169922.vib to the server operating system，such as the /tmp directory，and execute the driver installation command.

#esxcli software vib install -v /tmp/MSCC_bootbank_smartpqi_67.4252.0.103-1OEM.670.0.0.8169922.vib

- Reboot the system.

#reboot

- Check if the driver version has been successfully upgraded.

#esxcli software vib list|grep smartpqi

Driver Uninstallation

- Execute uninstallation command

#esxcli software vib remove -n smartpqi

- Reboot the system.

#reboot

- Check if the driver version has successfully uninstalled.

#esxcli software vib list|grep smartpqi