Installation Guide of PMC RAID Card Driver in VMware System

目录

1.	Precautions for operation	3

2.	Preparation before operation	3

3.	Driver Installation	3

4.	Driver Uninstallation	4

Precautions for operation

- This driver version is only applicable to VMware ESXI 7.0.

- This driver is suitable for PMC RAID cards.

- Driver upgrade requires server restart to take effect.

Preparation before operation

- Query the current operating system kernel.

#uname -a

- Query the current driver version. The driver version is 4149.0.5000.

#esxcli software vib list|grep smartpqi

Driver Installation

- After decompressing the driver package, upload the driver file MIS_bootbank_smartpqi_70.4330.0.116-1OEM.700.1.0.15843807.vib to the server operating system，such as the /tmp directory，and execute the driver installation command.

#esxcli software vib install -v /tmp/ MIS_bootbank_smartpqi_70.4330.0.116-1OEM.700.1.0.15843807.vib

- Reboot the system.

#reboot

- Check if the driver version has been successfully upgraded.

#esxcli software vib list|grep smartpqi

Driver Uninstallation

- Execute uninstallation command

#esxcli software vib remove -n driver name

- Reboot the system.

#reboot

- Check if the driver version has successfully uninstalled.

#esxcli software vib list|grep smartpqi